Postal Area Quality Check Tool

Quick Start Guide  |  ADP Team, TomTom

1. Overview

The Postal Area Quality Check Tool is a browser-based application that compares a Source postal area dataset against an MDS (Map Database Source) dataset and identifies geometric discrepancies. It runs five independent quality checks and exports results as GeoPackage, Shapefile, and/or Excel.

| Check | Output Layer | What It Finds |
| --- | --- | --- |
| Spike Detection | Spike_Output | Sharp vertices on MDS polygons (interior angle <= threshold) |
| Sliver Detection | Sliver_Suspicious_Area_Output | Tiny gap polygons between adjacent MDS features |
| Failed Geometry | Geometry_Failed_Less_Than_{N}m / Greater_Than_{N}m | Symmetric-difference parts outside the tolerance band |
| Multipart Validation | Multipart_Validation | Extra MDS parts (over-updated) and missing MDS parts (update missing) |
| Polygon Quality Check | Polygon_Quality_Check | UUID-matched pairs whose geometries differ by more than 5 m |

2. System Requirements

| Requirement | Details |
| --- | --- |
| Python | 3.10 or later. ArcGIS Pro Python (arcgispro-py3) is detected and preferred automatically. |
| Dependencies | streamlit, geopandas, shapely, fiona, pyogrio, pandas, openpyxl. Installed automatically on first launch. |
| Browser | Any modern browser (Chrome, Edge, Firefox). Opens automatically at http://localhost:8503. |
| Input formats | File Geodatabase (.gdb folder), Shapefile (.shp), GeoPackage (.gpkg) — for both Source and MDS. |
| Disk space | 500 MB or more free for large datasets and output files. |

3. Launching the Tool

Double-click the launcher batch file located in the tool folder:

C:\Prabhakar\PD_Quality\tool\Launch_PD_Quality_Checker.bat

The launcher will:

Detect ArcGIS Pro Python or fall back to system Python.

Install / update all required Python packages automatically.

Start the Streamlit server on port 8503.

Open http://localhost:8503 in your default browser after 15 seconds.

Note:  The command window must stay open while the tool is running. Closing it stops the server.

4. Step-by-Step Usage

Step 1 — Input Datasets

Click GDB to browse for a File Geodatabase folder, or SHP to select a Shapefile or GeoPackage. Do this for both Source and MDS. You can also type the path directly into the text box.

| Input Type | Browse Button | Layer Selection |
| --- | --- | --- |
| File Geodatabase (.gdb) | GDB — opens folder picker | Selectbox (1 layer) or multiselect (2+ layers). All layers selected by default; uncheck to exclude. |
| Shapefile (.shp) | SHP — opens file picker | Auto-detected. No selection needed — the file is processed directly. |
| GeoPackage (.gpkg) | SHP — opens file picker | Selectbox (1 layer) or multiselect (2+ layers). All layers selected by default. |

Note:  When a GDB or GPKG contains multiple layers with matching names in both Source and MDS (e.g. NL, DE, FR), the tool pairs them by name automatically. If names differ, layers are paired by position.

Step 2 — Quality Check Parameters

Configure the checks before running:

| Parameter | Default | Description |
| --- | --- | --- |
| Spike Angle Threshold | 15 deg | Maximum interior angle to flag as a spike vertex. FME default is 15 degrees. |
| Buffer Distance | 200 m | Split threshold for the Failed Geometry check. Parts < 1 m = noise; 1 m to Buffer = minor; > Buffer = major. |
| Spike Detection | On | Enable or disable the Step 1 spike check. |
| Sliver Detection | On | Enable or disable the Step 2 sliver check. |
| Failed Geometry | On | Enable or disable the Step 3 symmetric-difference check. |
| Multipart Check | On | Enable or disable the Step 4 multipart polygon comparison. |
| Polygon Quality | Off | UUID-based geometry matching. Off by default — enable for detailed per-feature comparison. |
| Include < 1 m noise layer | Off | When enabled, the Geometry_Failed_Less_Than_1m layer is written to output and shown in results. |

Warning:  If all five checks are unchecked, the tool re-enables all of them automatically.

Step 3 — Output Path

Click Browse to select the output folder. Choose a format:

Both (GeoPackage + Shapefile) — recommended; produces both formats in one run.

GeoPackage only — single .gpkg file containing all output layers.

Shapefile only — one .shp file per output layer inside a dated sub-folder.

Step 4 — Run

Click Run All QC Checks. A progress bar tracks each step. When processing multiple layers, the bar shows [1/3], [2/3], etc.

Note:  For large datasets (more than 500,000 features) the run may take several minutes. Do not close the browser tab or the command window during processing.

Step 5 — Results

After the run completes, a summary table and metric cards appear below the Run button. Each output layer has its own tab. Use the Filter rows text box inside any tab to search across all columns. Click Download Excel Report to save all layers to a single .xlsx file.

| Results Tab | Geometry Type | What to Look For |
| --- | --- | --- |
| Spike_Output | POINT | Each point is a spike vertex. _spike_angle column shows the measured angle. |
| Sliver_Suspicious_Area_Output | POLYGON | Tiny gap polygons between features. Check _sliver_area column. |
| Geometry_Failed_Less_Than_1m | POLYGON | Sub-1 m noise artefacts. Shown only when Include < 1 m noise layer is checked. |
| Geometry_Failed_Less_Than_{N}m | POLYGON | Minor deviations (1 m to buffer value). Investigate shared boundary shifts. |
| Geometry_Failed_Greater_Than_{N}m | POLYGON | Major deviations (> buffer value). High-priority errors requiring correction. |
| Multipart_Validation | POLYGON | Filter by _issue_type column: 'Extra in MDS' or 'Missing in MDS'. |
| Polygon_Quality_Check | POLYGON | UUID-matched sym-diff parts. Enabled only when Polygon Quality checkbox is on. |

5. Output File Naming

Output files are stamped with the run date and time (YYYYMMDD_HHMMSS):

QC_Results_20260701_143022.gpkg

QC_Results_20260701_143022_SHP\   (Shapefile folder)

QC_Results_20260701_143022.xlsx

Note:  When multiple GDB or GPKG layers are processed together, all layers are merged into one output file. A _layer column is added to every row to identify which source layer each error came from.

6. Common Issues & Solutions

Issue: Cannot read Source / Cannot read MDS

Possible causes and fixes:

Wrong path — verify the path in the text box exactly matches the folder or file on disk.

Wrong browse button — a GDB is a folder, use the GDB button. A .shp or .gpkg is a file, use the SHP button.

Missing driver — ensure fiona and pyogrio are installed. Re-run the launcher .bat to trigger a dependency update.

Issue: Step 3 skipped — no common ID column

The Failed Geometry check matches Source and MDS features by a shared ID column (e.g. uuid, CP5). If the two datasets use different column names, Step 3 is skipped and all three Geometry_Failed layers will be empty.

How to fix:

Rename the ID column in one dataset to match the other before running the tool.

Check both datasets in QGIS or ArcGIS Pro to confirm column names overlap.

Issue: Port 8503 already in use

If a previous session was not closed properly, port 8503 may still be occupied.

How to fix:

Open Task Manager, go to Details, find python.exe processes running streamlit, click End Task.

Alternatively, restart the computer and relaunch the .bat file.

To use a different port, open the .bat in Notepad and change --server.port 8503 to --server.port 8504 (or any free port), then open http://localhost:8504 manually.

Issue: Python not found

The launcher reports: ERROR: Python not found. Install Python 3.11+ or ArcGIS Pro.

How to fix:

Install Python 3.10+ from https://python.org/downloads and ensure 'Add Python to PATH' is checked during installation.

If ArcGIS Pro is installed, confirm the path C:\Program Files\ArcGIS\Pro\bin\Python\envs\arcgispro-py3\python.exe exists.

Issue: Missing geopandas / fiona after install

The .bat file runs pip install automatically, but pip may lack write permissions in a corporate environment.

How to fix:

Run the .bat as Administrator: right-click the file and choose Run as administrator.

Or install manually from a command prompt:

pip install geopandas fiona shapely pyogrio streamlit openpyxl

Issue: All output layers are empty (0 rows)

If the run completes but every count shows 0, check the following:

Source and MDS do not cover the same geographic area — open both in QGIS and verify they overlap.

No CRS defined — the tool re-projects to EPSG:4326 automatically, but if a dataset has no CRS assigned, the spatial match may fail. Assign a CRS in QGIS before running.

Wrong layer selected — for multi-layer datasets, confirm the correct layer is checked in the multiselect widget.

No common ID column — see the 'Step 3 skipped' issue above.

Issue: Road-parallel linear slivers in Failed Geometry

The tool automatically filters out road-parallel linear artefacts using three combined conditions (aspect ratio > 4, width < 30 m, length > 150 m). Only parts satisfying ALL three are suppressed, so short interior errors are preserved. If real errors are being filtered, adjust the constants _GEOM_LINEAR_ASPECT_RATIO, _GEOM_ROAD_WIDTH_MAX_M, and _GEOM_SLIVER_MIN_LENGTH_M in analyzer.py.

Issue: Browser does not open automatically

How to fix:

Open any browser manually and navigate to http://localhost:8503.

If the page does not load, wait another 30 seconds (large dependency installs can delay startup), then refresh.

Issue: Excel download produces a corrupted file

pip install openpyxl

Re-run the launcher .bat after installing; the dependency check at startup will pick it up.

Issue: Unexpected Extra / Missing counts in Multipart Validation

The multipart check performs a greedy 1-to-1 spatial overlap match between Source and MDS parts. Counts can be unexpected when:

Source and MDS split the same real-world polygon differently (e.g. 3 parts vs 4 parts).

Features are very close but not overlapping — try snapping datasets before running.

The MDS update is not yet complete — run the check only after the MDS update is finished.

7. Quick Reference Card

| Task | Action |
| --- | --- |
| Launch the tool | Double-click Launch_PD_Quality_Checker.bat |
| Open in browser manually | Navigate to http://localhost:8503 |
| Browse GDB folder | Click the GDB button next to Source or MDS |
| Browse SHP or GPKG file | Click the SHP button next to Source or MDS |
| Select specific layers | Use the multiselect widget; uncheck layers to exclude |
| Change buffer threshold | Edit the Buffer Distance field in Step 2 (default 200 m) |
| Disable a check | Uncheck the relevant checkbox in Step 2 |
| Enable < 1 m noise output | Check 'Include < 1 m noise layer' in Step 2 |
| Export results | Choose format in Step 3 then click Run |
| Download Excel report | Click 'Download Excel Report' in the Results section |
| Stop the tool | Close the command window |
| Restart after crash | End all python.exe (streamlit) processes in Task Manager, then relaunch .bat |

8. Contact & Support

For bug reports, feature requests, or questions, contact the ADP Team:

Prabhakar Chaudhari  —  prabhakar.chaudhari@tomtom.com

Postal Area Quality Check Tool  |  ADP Team, TomTom  |  Internal Use Only